10 August 2026

# Precise bidirectional force measurement with S-beam sensors

# New force sensors with integrated analogue electronics for nominal forces from 50 N to 10 kN

# MEGATRON is launching two new S-beam force sensors with integrated analogue amplifiers: The KT1405 and KT1505 measure switching and actuation forces between 50 N and 10 kN with the highest accuracy via a full-bridge strain gauge circuit. These force transducers enable users to achieve even more precise measurement results while saving installation space, as no external signal conditioning is required.

# The KT1405 S-Beam force sensor is ideal for applications involving small to medium force ranges, from 50 N to 1 kN. The KT1505, on the other hand, measures forces between 1 kN and 10 kN. Both sensors can measure compressive or tensile forces, and bidirectional compressive and tensile forces can be measured using just a single sensor if required. This is made possible by the integrated analogue electronics, which provide a bipolar voltage output ranging from −10 V to +10 V, in addition to the 0–10 V and 4–20 mA output signal options. The force transducer provides information on the direction of the force via this output: compressive forces generate a positive signal (0 to +10 V) and tensile forces generate a negative signal (0 to −10 V). MEGATRON has integrated analogue electronics into its force sensors in response to customer demand, as previously, these sensors could not be used due to the lack of a negative output voltage.

# The new S-Beam force sensors boast an impressive response time of under 1 ms and a cut-off frequency of 1,600 Hz. As they feature an analogue amplifier, there is no need for bulky external electronics. Despite their integrated electronics, the sensors are highly compact, measuring 70 mm x 64 mm x 34 mm (KT1405) and 80 mm x 71 mm x 40.5 mm (KT1505), respectively. Furthermore, their cabling requirements are significantly lower than those of sensors with a separate amplifier. All force sensors are calibrated and tested at the factory, and a test report documenting compliance with the specified measurement values for the selected nominal force is supplied.

# The KT1405 series of S-Beam force sensors is typically used in assembly, handling, dosing, packaging and testing systems. KT1505 series sensors are ideal for measuring force in production plants and equipment, as well as in dosing and packaging systems. They can also be used to monitor joining and assembly processes.

(2,321 characters incl. spaces)

About MEGATRON

MEGATRON Elektronik GmbH & Co. KG is a leading German supplier of precision sensors, industrial joysticks, precision resistors, small plastic parts and electronic housings. Owner-managed since 1960 and based in Putzbrunn near Munich, the company develops, manufactures and markets its own products and those of long-standing international partners throughout the world. Customised and economical product solutions are developed in close cooperation with OEM customers. A wide range of immediately available stock items completes the profile.

(541 characters incl. spaces)

Image

MEGATRON-S-Beam-sensors-KT1405-KT1505.jpg: MEGATRON’s new S-Beam sensors, KT1405 and KT1505, enable precise bidirectional force measurement

Image: MEGATRON Elektronik GmbH & Co. KG

Meta-Title:

Precise force measurement in confined spaces

Meta-Description:

The KT1405 and KT1505 S-beam force transducers from MEGATRON have a nominal force range of 50 N to 10 kN and a very compact design.

Keywords

Force sensors, load cells, sensor technology, measurement technology, analogue amplifiers, signal amplifiers, materials testing equipment, materials testing, packaging systems, packaging machines, MEGATRON Elektronik GmbH & Co. KG

Deeplinks:

https://www.megatron.de/en/

https://www.megatron.de/en/category/load-cells

https://www.megatron.de/en/products/s-beam-load-cells/s-beam-force-transducer-kt1405

https://www.megatron.de/en/products/s-beam-load-cells/s-beam-load-cell-kt1505

Social Media

LinkedIn profile: https://de.linkedin.com/company/megatron-elektronik-gmbh-co-kg

Linking the company on LinkedIn: @MEGATRON Elektronik GmbH & Co. KG

Download-Area

https://www.koehler-partner.de/pressezentrum/megatron

Contact:

MEGATRON Elektronik GmbH & Co. KG

Hermann-Oberth-Str. 7 ▪ 85640 Putzbrunn / München

Tel.: +49 89 46094-0 ▪ Fax: +49 89 46094-201

info@megatron.de ▪ www.megatron.de

Press Service:

Köhler + Partner GmbH

Brauerstraße 42 ▪ 21244 Buchholz i.d. Nordheide

Tel. +49 4181 92892-0 ▪ Fax +49 4181 92892-55

info@koehler-partner.de ▪ www.koehler-partner.de